| Civil Rights Compliance Review Facility Checklist |  |
| --- | --- |
| School District |  |
| Completed by: |  |
| Building: |  |
| Date this form was completed: | Date of construction of main building |
| Note: One Facility Checklist must be completed for each building with CTE classes or administrative personnel who meet with CTE students. Each facility parking lot is considered a separate facility – start date(s) of construction, resurfacing, painting, and number of parking spaces available are needed. |  |

Alterations/Renovations to Primary Function Areas in  this Building*

| Physical location of alteration/ renovation | Work done | Date work started | Applicable Architectural Standard |
| --- | --- | --- | --- |
| Note for each parking lot: # of parking spaces # of accessible spaces # of van accessible spaces Date of last paving: Date of last striping | : |  |  |

- Only alterations and/or renovations started after June 4, 1977, affecting usable space, e.g., classrooms, assembly areas, office areas, need be noted.

- Alterations and/or renovations to HVAC, boilers, electrical, painting, plumbing etc., need not be noted.

- If physical construction or alterations commence on or after March 15, 2012, then new construction and alterations shall comply with the 2010 Standards

| Accessible Routes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Is there an accessible route connecting accessible entrances with all accessible elements and spaces within the building? |  |  |  |  | 206; |
| Is the accessible route at least 36” wide except at doorways where it can narrow to 32” clear. |  |  |  |  |  |
| Where the accessible route makes a U-turn around an obstacle which is less than 48” wide, does the pathway width increase to at least 42” on the approaches and 48” in the turn? |  |  |  |  |  |
| If the accessible route is less than 60” wide, are there passing spaces at least 60” wide and 60” long or intersecting corridors allowing passing at reasonable intervals not exceeding 200’? |  |  |  |  |  |
| If objects mounted to the wall have leading edges between 27”-80” from the floor, do they project less than 4” into the pathway? (Wall mounted objects with leading edges at or below 27” may project any amount so long as the required clear width of an accessible route is not reduced.) |  |  |  |  |  |
| Do free-standing objects mounted on posts with leading edges between 27”-80” high project less than 12” into the perpendicular route or travel? |  |  |  |  |  |
| Is there an accessible path at least 36” clear alongside the protruding object? |  |  |  |  |  |
| Is there at least 80” clear head room on an accessible route? |  |  |  |  |  |
| If there is less than 80” clear head room in an area adjoining an accessible route, is there a cane detectable barrier within 27” of the floor? |  |  |  |  |  |
| Are the floor surfaces on accessible routes stable, firm, and slip-resistant? |  |  |  |  |  |
| Is the slope of the accessible route no greater than 1:20? OR Where the slope is greater than 1:20, does it comply with the requirements for ramps? |  |  |  |  |  |
| Is the cross-slope no greater than 1:48? |  |  |  |  |  |
| Are ramps or elevators used for changes in level greater than ½”? (Lifts may only be used in certain limited situations in new construction) |  |  |  |  |  |
| When walkway levels change, is the vertical difference less than ¼”? OR Are changes in level between ¼”- ½” beveled with a slope no greater than 1:2? [45 degrees bevel] |  |  |  |  |  |
| Is there directional signage to the accessible entrance? |  |  |  |  |  |
| Accessible Routes p. 2 |  |  |  |  |  |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| When floor materials change, does the vertical difference between them meet the above requirements? ½” bevel or ramp |  |  |  |  |  |
| If carpet or carpet tile is used on the floor, is it securely attached? |  |  |  |  |  |
| Are exposed edges of carpet fastened to the floor and have trim along their entire length? |  |  |  |  |  |
| Is it a low pile type of carpet (½” maximum) with a firm pad or no pad underneath it? |  |  |  |  |  |
| Does each door that is an element of an accessible route comply with 4.13? |  |  |  |  | 4.13 |
| If a revolving door or turnstile is used on an accessible route, is an accessible door or gate provided adjacent to the revolving door or turnstile to facilitate the same use pattern? |  |  |  |  |  |
| Do gates, including ticket gates, meet all the applicable specifications of 4.13? |  |  |  |  |  |
| Do signs which provide direction to, or information about, functional spaces of the building comply with 4.30.2, 4.30.3, and 4.30.5? Exception: Building directories, menus, and all other signs which are temporary are not required to comply. |  |  |  |  |  |
| Do signs which identify permanent rooms and spaces comply with standards? Brailled |  |  |  |  |  |
| If emergency warning systems are provided, do they include both audible alarms and visual alarms complying with standards? |  |  |  |  |  |

| Assembly Areas |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Does at least one door into each public space have at least a 32” clear opening? |  |  |  |  | 221 |
| Are appropriate maneuvering clearances provided at accessible doors? 18” clear adjacent to the latch side of the door |  |  |  |  |  |
| Can doors be opened without too much force (5 lbf max for interior doors)? |  |  |  |  |  |
| Are door handles 48 inches high or less and operable without tight grasping, pinching or twisting of the wrist? |  |  |  |  |  |
| Are all thresholds level (less than ¼”) or beveled, with a slope no greater than 1:2 up to ½ inch high? 45 degree bevel |  |  |  |  |  |
| If there are sliding doors, is the operating hardware exposed and usable from both sides when the doors are fully open? Track can be ¾” high. |  |  |  |  |  |
| In assembly areas with fixed seating, is the required number of wheelchair locations provided? Horizontal and vertical dispersion? Aisle seats? |  |  |  |  | 221.2.3.1 221.2.3.2 |
| Is each wheelchair space at least 48” deep for approach from the front or rear, and 60” deep for approach from the side? WHEELCHAIR FOOTPRINT IS 48” x 30” |  |  |  |  |  |
| Do wheelchair spaces adjoin an accessible route that also serves as an accessible means of egress? |  |  |  |  |  |
| Is at least one companion fixed seat provided next to each wheelchair seating area? |  |  |  |  | 221.3 |
| Does an accessible route connect wheelchair seating locations with performing areas, including stages, arena floors, dressing rooms, locker rooms, and other spaces used by performers? |  |  |  |  |  |
| In assembly areas, where audible communications are integral to the use of the space, are an adequate number of assistive listening systems provided? |  |  |  |  |  |
| Is there signage indicating the availability of assistive listening devices? |  |  |  |  |  |
| Cafeterias |  |  |  |  |  |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010ADA Standards |
|  | Y | N | NA |  |  |
| Does at least one door into each public space have at least a 32-inch clear opening? |  |  |  |  |  |
| Are appropriate maneuvering clearances provided at accessible doors? |  |  |  |  |  |
| Can doors be opened without too much force (5lbf max for interior doors)? |  |  |  |  |  |
| Are door handles 48” high or less and operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Are all thresholds level (less than ¼”), or beveled, with a slope no greater than 1:2 up to ½” high? |  |  |  |  |  |
| If there are sliding doors, is the operating hardware exposed and usable from both sides when the doors are fully open? |  |  |  |  |  |
| Are at least 5%, but a minimum of 1, of the fixed tables accessible? |  |  |  |  |  |
| Is the knee space at accessible tables at least 27” high, 30” wide and 19” deep? |  |  |  |  |  |
| Is the top of each accessible table or counter between 28”-34” above the floor? |  |  |  |  |  |
| Where possible, are the accessible tables or counters, distributed throughout the space? |  |  |  |  |  |
| Are all aisles between accessible fixed tables at least 36” wide? |  |  |  |  |  |
| Where counter service is provided, is there at least a 60” long portion of the main counter that is no more than 34” high, and that has at least 27” of knee space below, or is service available at accessible tables within the same area? |  |  |  |  |  |
| Do food service lines have at least 36” clear width? |  |  |  |  |  |
| Are tray slides no more than 34” above the floor? |  |  |  |  |  |
| Are at least 50% of self-service food service shelves 15”-48” for perpendicular approach and 9”-54” for a parallel approach? |  |  |  |  | 2010 Standards Reach Ranges for both forward and side approach are from 15” above floor to 48” [AFF = above finished floor] |
| Are self-service shelves and dispensing devices for tableware, dishware, condiments, food, and beverages installed 15”-48” for forward approach, or 9”-54” for a parallel approach? |  |  |  |  |  |
| Do self-service shelves and vending machines have at least a 30”x48” adjacent clear floor space? |  |  |  |  |  |
| Are vending machines on an accessible route? |  |  |  |  |  |

| Elevators |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Does the elevator have automatic operating and self-leveling features? |  |  |  |  |  |
| Are there both visible and verbal or audible door opening/closing and floor indicators (one tone = up; two tones = down)? |  |  |  |  |  |
| Are the call buttons in the hallway at least ¾” wide, and centered at 42” AFF? |  |  |  |  |  |
| Do hall call buttons have visual signals to indicate when each call is registered and answered? |  |  |  |  |  |
| Is there a sign on the jamb at each floor identifying the floor in raised and Braille letters? |  |  |  |  |  |
| Is the door opening at least 36” wide? |  |  |  |  |  |
| For a centered opening, is the minimum inside dimension of elevator cars 51”x80”? |  |  |  |  |  |
| For an off-center opening, is the minimum inside dimension of elevator cars 51”x68”? |  |  |  |  |  |
| Are car control buttons no higher than 48” for forward reach and 54” for side reach? |  |  |  |  |  |
| Do the controls inside the cab have raised and Braille lettering? |  |  |  |  |  |
| Are the emergency controls grouped at the bottom of the control panel and centered no less than 35” above the floor? |  |  |  |  |  |
| If an emergency intercom is provided, is it usable without voice communication? |  |  |  |  |  |
| Is the emergency intercom identified by Braille and raised letters and raised symbols? |  |  |  |  |  |
| If the communication system is in a closed compartment, is the hardware on the compartment operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |

| Entrances |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Are at least 50% of all public entrances accessible? Is at least one accessible entrance a ground floor entrance? |  |  |  |  |  |
| Do all inaccessible entrances have signs indicating the location of the nearest accessible entrance? |  |  |  |  |  |
| If not all entrances are accessible, are the accessible entrances identified by the international symbol of accessibility? |  |  |  |  |  |
| Does at least one door at each accessible entrance have at least 32” clear opening (for a double door, at least one 32” leaf)? |  |  |  |  |  |
| Are appropriate maneuvering clearances provided at accessible doors? |  |  |  |  |  |
| Is the threshold level (less than ¼” high) or beveled with a slope no greater than 1:2 up to ½” high (3/4” max for exterior sliding doors)? |  |  |  |  |  |
| Are door handles at accessible entrances no higher than 48” and operable with one hand and without tight grasping, pinching or twisting of the wrist? |  |  |  |  |  |
| If there is a revolving door or turnstile at an entrance, is there an accessible door or a gate next to it? |  |  |  |  |  |
| On sliding doors, is the operating hardware exposed and usable from both sides when the doors are fully opened? |  |  |  |  |  |
| Can accessible doors be opened without too much force (max 5lbf for interior doors)? |  |  |  |  |  |
| If the accessible doors have closers, do they take at least 3 seconds to close to a point 3” from the latch? |  |  |  |  |  |

| Exterior Route of Travel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Is there at least one accessible route of travel from public transportation stops, accessible parking spaces, accessible passenger loading zones, public streets, and sidewalks to the accessible entrance? |  |  |  |  |  |
| Is there at least one accessible route that connects accessible buildings, facilities, elements and spaces that are on the same site? |  |  |  |  |  |
| Is the accessible route(s) stable, firm and slip-resistant? |  |  |  |  |  |
| Is the accessible route at least 36” wide? |  |  |  |  |  |
| If the accessible route(s) is less than 60” wide, are there passing spaces at least 60”x60”, or T-intersection of corridors, located at reasonable intervals but not more than 200’ apart? |  |  |  |  |  |
| Is there at least 80” of clear head room on every route? If an area adjoining an accessible route has less than 80” of clear head room, is a barrier to warn persons with visual impairments provided? |  |  |  |  |  |
| Are all obstacles along routes of travel cane-detectable (located within 27” of the ground or higher than 80”, or protruding no more than 4” into the route of travel)? |  |  |  |  |  |
| If gratings are located in walking surfaces, are the openings of the grating no greater than ½” wide in one direction? Are the long dimensions of rectangular openings placed perpendicular to the dominant direction of travel? |  |  |  |  |  |
| Is the cross-slope of the accessible route(s) no greater than 1:48? |  |  |  |  |  |
| Is the running-slope of the accessible route no greater than 1:20, or is there an accessible ramp if the slope is greater than 1:20 (use ramp survey)? |  |  |  |  |  |
| Are walkway level changes no more than ¼”, or if they are between ¼” and ½”, are they beveled with a slope no greater than 1:2? |  |  |  |  |  |
| Exterior Route of Travel Page 2 |  |  |  |  |  |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Are there curb cuts, ramps, platform lifts or elevators where there is a change in level greater than ½”? |  |  |  |  |  |
| Is there a curb cut wherever an accessible route crosses a curb? |  |  |  |  |  |
| Is the slope of the curb ramp no greater than 1:12? If there is not enough space to use a 1:12 slope or less, is the slope of the curb ramp no greater than 1:10 for a maximum rise of 6”, or 1:8 for a maximum rise of 3”? |  |  |  |  |  |
| Are maximum slopes of adjoining gutters, immediately adjacent road surface, or accessible route no greater than 1:20? Is the transition from the curb ramp to adjoining surfaces flush and free of abrupt changes? |  |  |  |  |  |
| If curb ramp is located where pedestrians must walk across the ramp, or where it is not protected by handrails or guardrails, does it have flared sides with a maximum slope of 1:10? |  |  |  |  |  |
| Is the width of the curb ramp, not including the flared wises, no less than 36 inches? |  |  |  |  |  |
| Is the surface of the curb ramp stable, firm and slip-resistant? |  |  |  |  |  |

| Libraries |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments |  |
|  | Y | N | NA |  |  |
| Does at least one door into each public space have at least a 32” clear opening? |  |  |  |  |  |
| Are appropriate maneuvering clearances provided at accessible doors? |  |  |  |  |  |
| Can doors be opened without too much force (5lbf max for interior doors)? |  |  |  |  |  |
| Are door handles 48” high or less, and operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Are all thresholds level (less than ¼”), or beveled, with a slope no greater than 1:2 up to ½” high? If there are sliding doors, is the operating hardware exposed and usable from both sides when the doors are fully open? |  |  |  |  |  |
| If there is a turnstile or security device, is there an accessible door or gate adjacent to it? |  |  |  |  |  |
| Is at least one lane for each checkout area accessible? |  |  |  |  |  |
| Are at least 5% (but a minimum of one) of fixed seating, tables or study carrels accessible? |  |  |  |  |  |
| Is there a clear floor space of 30”x 48” for wheelchair spaces at accessible tables? |  |  |  |  |  |
| Are the tops of the accessible fixed tables between 28” -34” above the floor? |  |  |  |  |  |
| Is there knee space under the accessible, fixed tables at least 27” high, 30” wide and 19” deep? |  |  |  |  |  |
| Are the aisles leading up to and between accessible tables and carrels at least 36” wide? |  |  |  |  |  |
| Is the clear aisle space at card catalogs and magazine displays at least 36” wide? |  |  |  |  |  |
| Is the minimum clear aisle width between stacks at least 36”? |  |  |  |  |  |
| Are card catalogs between 18”-54” high? |  |  |  |  |  |

| Lifts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| If platform lifts are used, can a person using a wheelchair enter, operate, and exit the lift without assistance? |  |  |  |  |  |
| Is the platform lift at least 30” x 48”? |  |  |  |  |  |
| Is there at least 30” x 48” of clear space for a person using a wheelchair to approach to reach the controls and use the lift? |  |  |  |  |  |
| If there is a door on the lift, is the door accessible? |  |  |  |  |  |
| Are controls between 15” x 48” high (up to 54” if a side approach is possible)? |  |  |  |  |  |
| Are the controls operable with one hand, and without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Lobbies and Corridors |  |  |  |  |  |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Does an accessible entrance connect with all accessible elements and spaces in the building? |  |  |  |  |  |
| Is the accessible route to all public spaces at least 36” wide? If the accessible route turns around an obstruction less than 48” wide, is the route at least 42” wide on the approach to and exit from the turn, and at least 48” wide at the base of the turn? |  |  |  |  |  |
| Is the cross-slope of the accessible route no steeper than 1:48? |  |  |  |  |  |
| If the accessible route is less than 5’ wide, are there passing spaces 5’ x 5’, or T-intersecting corridors located at reasonable intervals no more than 200’ apart? |  |  |  |  |  |
| Is there at least 80” of clear head room on every route? If an area adjoining an accessible route has less than 80” of clear head room, is a barrier to warn persons with visual impairments provided? |  |  |  |  |  |
| Are floors on an accessible route stable, firm, and slip-resistant? |  |  |  |  |  |
| Is the slope no more than 1:20, or is there a ramp when the slope is greater than 1:20? |  |  |  |  |  |
| If objects mounted to the wall have leading edges between 27” – 80” from the floor, do they project no more than 4” into the route of travel? |  |  |  |  |  |
| Are walkway level changes less than ¼”, or, if they are between ¼” – ½”, are they beveled with a slope no greater than 1:2? |  |  |  |  |  |
| Are ramps provided for changes in level greater than ½”? |  |  |  |  |  |
| Does at least one door into each public space have at least a 32” clear opening? |  |  |  |  |  |
| Are appropriate maneuvering clearances provided at accessible doors? |  |  |  |  |  |
| Can doors be opened without too much force (5 lbf max for interior doors)? |  |  |  |  |  |

| Parking |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| How many parking spaces are there in the main parking lot? Is there an adequate number of accessible parking spaces available in this lot? (1:25 spaces) How many spaces are van accessible? Are there upright signs for each space? Are the accessible spaces the ones closest to an accessible entrance? |  |  |  | Main lot # spaces: _________ Main lot # HA spaces: _______ Date last paved? __________ Date of last striping? ________ | 216 |
| How many parking spaces are there in other parking lots? Lot 1 location: _________________________ e.g., east side of building, etc. Is there an accessible entrance nearby? Are the accessible spaces the ones closest to this accessible entrance? Lot 2 location: __________________________ Is there an accessible entrance nearby? Are the accessible spaces the ones closest to this accessible Lot 3 location: __________________________ Is there an accessible entrance nearby? Are the accessible spaces the ones closest to this accessible entrance? |  |  |  | Additional parking lots: Lot 1 # spaces: _________ Lot 1 # accessible spaces: ________ ________ Van Accessible Lot 2 # spaces: ________ Lot 2 accessible spaces: ________ ________ Van Accessible Lot #3 spaces: _________ Lot # 3 accessible spaces: ________ _______ Van Accessible | 208 208.3 216 |
| Are accessible parking spaces at least 8’ wide, with a 5’ access aisle? (Two spaces can share an access aisle) |  |  |  |  |  |
| Is one in every 6 spaces, but at least one, van-accessible with a 96” wide access aisle, and 98” of vertical clearance? |  |  |  |  |  |
| Are the access aisles part of the accessible route to an accessible entrance? |  |  |  |  |  |
| Is the slope of the accessible parking area and access aisle no more than 1:48 |  |  |  |  |  |
| Are accessible spaces marked with a vertical sign showing the international symbol of accessibility? |  |  |  |  |  |
| Is there an access aisle 60” wide by 20’ long adjacent and parallel to the vehicle pull-up space? |  |  |  |  |  |
| If there is a curb between the access aisle and the pull-up space, is there a curb ramp? |  |  |  |  |  |
| If a walkway crosses or adjoins the driveway and there is no curb, does the walkway edge have a detectable warning surface? |  |  |  |  |  |

| Ramps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Are the slopes of the ramp no greater than 1:12? |  |  |  |  |  |
| Is the rise of each ramp run (vertical distance between landings) no greater than 30”? |  |  |  |  |  |
| Is the cross slope no greater than 1:48? |  |  |  |  |  |
| Do all ramps longer than 6’ or with more than 6” rise have handrails on both sides? |  |  |  |  |  |
| Are railings continuous, sturdy, and between 34”-38” high? |  |  |  |  |  |
| Is the width between railings or curbs at least 36”? |  |  |  |  |  |
| Are ramps stable, firm and slip-resistant? |  |  |  |  |  |
| Is there a 5’ long level landing at every 30’ horizontal length of ramp, at the top and bottom of every ramp and where the ramp changes direction? |  |  |  |  |  |
| Do ramps and landings with drop-offs have walls, railings, projecting surfaces, or curbs at least two inches high to prevent people from slipping off the ramp? 12” side extension or bar along the ramp 4” above ramp surface. |  |  |  |  |  |

| Rooms and Spaces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Are all aisles and pathways to materials and services at least 36” wide? |  |  |  |  |  |
| If aisles between fixed furniture are less than 5’ wide, are there passing spaces 5’ x 5’ or intersecting aisles at reasonable intervals not exceeding 200’ maximum? |  |  |  |  |  |
| Are floors stable, firm and slip-resistant? |  |  |  |  |  |
| Is carpeting low-pile, tightly woven, and securely attached? |  |  |  |  |  |
| In routes through public areas are all obstacles cane-detectable (located within 27” of the floor or higher than 80” or protruding not more than 4” from the wall)? |  |  |  |  |  |
| Is there at least 80” of clear head room on every route? If an area adjoining an accessible route has less than 80” of clear head room, is a barrier to warn persons with visual impairments provided? |  |  |  |  |  |
| Does at least one door into each public space have at least a 32” clear opening? |  |  |  |  |  |
| Are appropriate maneuvering clearances provided at accessible doors? |  |  |  |  |  |
| Can doors be opened without too much force (5lbf maximum for interior doors?) |  |  |  |  |  |
| Are door handles 48” high or less, and operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Are all thresholds level (less than ¼” ) or beveled, with a slope no greater than 1:2 up to ½” high? |  |  |  |  |  |
| If there are sliding doors, is the operating hardware exposed and usable from both sides when the doors are fully open? |  |  |  |  |  |
| Do signs designating permanent rooms and spaces, such as rest room signs, signs at exit doors, and room numbers, comply with the appropriate requirements for signage? |  |  |  |  |  |
| Are all controls that are available for use by the public (including electrical, mechanical, cabinet, game, and self-service controls) located between 15” – 48” for forward reach and between 9”-54” for side reach? |  |  |  |  |  |
| Are they operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| If fixed or built-in seating or tables are provided in accessible public or common use areas, do at least 5%, but not less than one of the fixed or built-in seating areas or tables provide 30” x 48” of clear floor space? |  |  |  |  |  |
| Are the aisles between fixed seating at least 36” wide? |  |  |  |  |  |
| Are the tops of at least 5% of fixed tables or counters between 28”-34” high? |  |  |  |  |  |
| Are knee spaces at accessible fixed tables at least 27” high, 30” wide and 19” deep? |  |  |  |  |  |
| Are at least 50% of drinking fountains, but at least one, on each floor accessible? |  |  |  |  |  |
| Do the accessible wall-and post-mounted cantilevered units have clear knee space 27” high, 30” wide, and 17”-19” deep? Do these units have a minimum clear floor space 30” x 48” to allow a person who uses a wheelchair to approach the unit facing forward? |  |  |  |  |  |
| Do the accessible free-standing or built-in-drinking fountains not having a clear space under them have clear floor space of at least 30” x 48” in front to allow a parallel approach to the unit? |  |  |  |  |  |
| Do the accessible drinking fountains have spouts no higher than 36”? |  |  |  |  |  |
| Are controls on accessible drinking fountains mounted on the front or on the side near the front edge, and operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Do alarms have both visible and audible signals? |  |  |  |  |  |

| Stairs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Do stairs have closed risers? |  |  |  |  |  |
| Are stair treads no less than 11”? |  |  |  |  |  |
| Do stairs have continuous handrails on both sides, with extensions beyond the top and bottom stairs? |  |  |  |  |  |
| Do nosings project no more than 1-1/2”? |  |  |  |  |  |

| Toilet Rooms: Overall Facility |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| If rest rooms are available to the public, is at least one rest room on each floor that has public restrooms (either one for each sex, or unisex) on an accessible route and fully accessible? |  |  |  |  | 213 |
| Are there signs at inaccessible restrooms that give directions to accessible ones? |  |  |  |  |  |
| When not all toilet facilities are accessible, are accessible toilet rooms identified by the international symbol of accessibility? |  |  |  |  |  |
| Is there at least one wheelchair-accessible stall that is at least 5’ wide, clear of the door swing, and at least 56” long if the toilet is wall-mounted? If it is technically infeasible to provide such a standard stall, is there a stall that is either 36” x 66” or 48” x 66” if the toilet is wall-mounted, or either 36” x69”, or 48” x 69” if the toilet is floor-mounted? |  |  |  |  |  |

| Toilet Rooms: Accessible Restroom # ___________________ (copy this form for as many accessible restrooms as there are in the facility; complete one form per restroom) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Do doors have at least a 32” clear opening? |  |  |  |  | 213 |
| Are appropriate maneuvering clearances provided at doors? |  |  |  |  |  |
| Can doors be opened without too much force (5lbf maximum for interior doors?) |  |  |  |  |  |
| Are door handles 48” high or less and operable without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Are all thresholds level (less than ¼”) or beveled, with a slope no greater than 1:2 up to ½” high? |  |  |  |  |  |
| Is there a tactile signage identifying the restrooms placed on the wall at the latch side of the door, centered 60” inches above the floor? |  |  |  |  |  |
| Is there a 5’-diameter clear space or a T-shaped space in the restroom to make turns? |  |  |  |  |  |
| Are all fixtures on an accessible route? |  |  |  |  |  |
| Can the door to the accessible toilet stall be operated without twisting or fine movement, on both the inside and outside? |  |  |  |  |  |
| Do the accessible toilet stalls have a minimum door opening of at least 32”? |  |  |  |  |  |
| Are there accessible grab bars in accessible toilet stalls? |  |  |  |  |  |
| Are there accessible grab bars at accessible water closets not located in stalls? |  |  |  |  |  |
| Are the accessible toilet seats 17”-19” high? |  |  |  |  |  |
| Are the flush controls on accessible toilets no higher than 44” and mounted on the wide side of toilet areas? |  |  |  |  |  |
| Is the toilet paper dispenser at least 19” above the floor? And between 7”-9” in front of the toilet? |  |  |  |  |  |
| If the toilet paper dispenser is above the side grab bar – is it at least 12” clear above it? |  |  |  |  |  |
| If the toilet paper dispenser is below the side grab bar – it is at least 1 ½ “clear beneath it? |  |  |  |  |  |
| Accessible Restroom p.2 (Room # __________________) |  |  |  |  |  |
| Questions | In compliance? |  |  | Dimensions/Comments | 2010 ADA Standards |
|  | Y | N | NA |  |  |
| Does one lavatory have a 30” wide by 48” deep clear space in front with a maximum of 19” of the depth under the lavatory? |  |  |  |  |  |
| Is the lavatory rim no higher than 34” from the floor? |  |  |  |  |  |
| Is there at least 29” from the floor to the bottom of the lavatory apron? |  |  |  |  |  |
| Is there at least 8” of clearance toward the wall provided for knee clearance? |  |  |  |  |  |
| Is there a maximum of 6” of clearance outward from the wall provided for toe clearance? |  |  |  |  |  |
| Can the faucet be operated with one hand and without tight grasping, pinching, or twisting of the wrist? |  |  |  |  |  |
| Are hot water pipes and drain pipes insulated, or configured to avoid contact with the legs of a person using a wheelchair? |  |  |  |  |  |
| Are soap and other dispensers and hand dryers no higher than 48” for forward reach, or 54” for side reach? 2010 ADA Standards prefer 48” high reach for both forward and side reach – and when over deep counters > 24” the height to operable parts drops to 46”. |  |  |  |  |  |
| Can they be operated with one hand and without twisting or fine movement? |  |  |  |  |  |
| Is there clear floor space of 30” x 48” in front of the dispensers? |  |  |  |  |  |
| Is the mirror mounted with the bottom edge of the reflecting surface no higher than 40”? |  |  |  |  |  |
| If alarms are provided in the restroom, do they have both visual and audible signals? |  |  |  |  |  |
| If drain and hot water pipes are exposed beneath the lavatory of at least one accessible lavatory, are those out the way or do they have pipe insulation installed? |  |  |  |  |  |
| Is the accessible route maintained near doorways such that trash receptacles are not hindering passage? |  |  |  |  |  |